FYS 4250

20 questions to each chapter

Chapter 1

- Can you make a simple sketch of a simple ear-thermometer and explain the principles of function?

- Can you make a simple sketch of a kidney stone crusher and explain the principles of function?

- What are the main risks for the patient when using a kidney stone crusher, and how can these be minimized?

- What is a cochlea implant and how does it work?

- How is the human ear able to discriminate between different frequencies (Frequency tuning) and how is the cochlea implant able to do the same discrimination?

- Understanding speech is the ultimate goal and the younger the patient, the more effective the cochlea implant will be. Why?

- What is a cochlea implant and how does it work?

- What requirements are typical for a cochlea implant amplifier

- Will a cochlea implant be effective if the patient has an injury in the cochlear nerve?

- What is a demand pacemaker and how is it designed?

- When a patient has a problem with the AV-node and intermittent discontinuation in the sinus node?

- What is the advantages/disadvantages with a unipolar vs a bipolar pacemaker?

- Most modern pacemakers are bipolar, why?

- What are the two main types of electrodes, and what is the difference between them?

- Why is polarization a topic when it comes to pacemaker electrodes?

- What is a half-cell potential and what are the theoretical types of electrodes?

- Describe the basic components of the half-cell overpotential

- When the frequency of a measured signal is increasing, how will that influence the half-cell potential?

- Why cannot regular hemodialysis be used to clean the blood from toxins during a liver failure?

Chapter 2

- Can you describe the circulatory system, the main parts of it and the function of the most important parts?

- How are the different parts of the heart synchronized?

- If we measure an elevated pressure in the right atrium, what could possibly be wrong and which devices could we use to fix the problem?

- What is an action potential and why is the action potential limited in time?

- What is the main purpose of the AV-node, and what happens if the SA-node is not working as expected?

- How is it possible to diagnose the heart by measuring electricity and what kind of equipment do you need?

- Why is the ECG-repolarization positive?

- What is an Einthoven triangle and how is it used?

- What is carrying the current in the body, and which problems can occur when we will transfer this current to cables outside the body?

- What is the voltage and frequency-range for ECG-signals?

- When the frequency-range of an ECG-device is in the range of 0-100 Hz, what would the ECG-signal look like on the monitor?

- What is the Wilson-terminal, what is the advantage and what is the disadvantage with the method?

- What are augmented leads, what is the advantage and how is the lead vector different from the regular Einthoven triangle?

- What is CMRR in an ECG-amplifier and why is it important?

- How can you minimize the noise from magnetic field pickup when acquiring the ECG-signals?

- Make a simple sketch of an ECG-amplifier and discuss briefly the main parts of it

- Describe a method for actively suppressing electromagnetic noise in an ECG-setup

- Is it a built-in electrical safety system in driven right leg?

- How many leads do you need for a 12-lead ECG, where are they located and which electrodes belong to which leads?

- Why is it almost impossible to design an ECG-monitor without using operational amplifiers?

- Why is it important that the ECG-system does not "load" the patient (= there is no current from the patient through the electrodes)

Chapter 3

- Describe what blood pressures you would expect from an arterial blood pressure monitor, and which you would expect inside the different parts of the heart

- What is the difference between invasive blood pressure measurements and non-invasive measurements?

- Describe the most common non-invasive methods for blood pressure measurements and their main advantages/disadvantages and sources of error

- Which of the invasive methods are able to measure absolute pressure, and which can measure relative pressure?

- How will an air bubble affect the blood pressure measurement from an invasive blood pressure measurement system?

- How can a wheatstone-bridge be useful for blood pressure measurements?

- What are the main advantages and disadvantages with intra-vascular blood pressure measurements?

- How can you measure the system characteristics of an invasive blood pressure system?

- What are the main sources of error for a wrist blood pressure measurement system, and what are the advantages?

- What is a stethoscope, how does it work and what is a typical frequencyresponse?

- How can you measure the blood pressure by means of a stethoscope, a sphygmomanometer and a cuff?

- Why is an automatic bloodpressure monitor with a sphygmomanometer, cuff and a microphone a bad idea?

- Why is it impossible to measure the diastolic pressure with an oscillometric method?

- What are the typical error sources for sphygmomanometers?

- EXPERT: Which non-invasive blood pressure measurement method would you recommend for measuring the blood pressure of a patient suffering from muscle-atrophy?

- What kind of device would you use for diagnosing glaucoma?

- How do you know that you have applied the right pressure to a manual tonometer?

- What are the basic principles of an automatic contact-tonometer?

- Which tonometry-method has the least risk of patient injury?

- What are the basic principles of an automatic non-contact tonometer?

Chapter 4

- What is the oxyhemoglobin dissociation curve, and what is the implication for the patient?

- What is saturation, what is the unit and how can we calculate it?

- Describe the cycle of breathing, what happens?

- If a person inhales more than the regular resting volume of 0,5 liter of air in the same length of time, would intra-alveolar pressure increase or decrease (become more negative)?

- Make a sketch of a pH electrode, describe the different parts and what requirements would you need from an amplifier?

- What is a fiberoptic pH-sensor, and what are the major advantages/disadvantages/sources of error for this?

- Why do we use green and red light for optical absorbance using pH sensitive dyes?

- What is a fluorescent dyesensor, and what do we mean with "irreversible" in this context?

- How do we read out the pH from a fluorescent dyesensor?

- Describe the main parts and function of a pCO2 electrode, and mention the most important sources of error

- Why is the core of the pCO2-electrode made of a pH-electrode?

- Describe the pO2 electrode, the main parts and the function and sources of error

- Why do we need a voltage source in the pO2 electrode, and why is it 0,7 Volt?

- Describe a simple system for measuring saturation, and explain how pulse oximetry is made

- How can we be sure that a pulse oximeter does not measure saturation in venous blood?

- If you would like to measure the blood flow with a thermistor, why is it a bad idea to have constant current through the resistor?

- Show a simple circuit that can provide constant temperature in a thermistor measuring the blood flow, and explain how you compensate for temperature variations in the blood.

- How can you diagnose a venous thrombosis (a clot in the vene) in a leg without imaging- or invasive methods?

- What are the main assumptions that is the basis for impedance plethysmography

- What are the main problems with a two-electrode measurement?