FYS 4250

Chapter 11

- How can we see typical signs of a myocardial infarction in an ECG-diagram?

- How many electrodes do you need for a 12-lead ECG?

- Explain the differences between the 12 leads

- What is a Percutaneous Coronary Intervention (PCI) procedure?

- Describe briefly the principle of an IABP (Intra arterial balloon pump)

- What are the main risks of an IABP and what can be used to reduce the risks?

- Which physical law describes the blood flow through a blood vessel, and how will a reduction of 50% in the diameter of a blood vessel influence on the flow?

- What is the main risks of using anti coagulants in patients?

- How can we measure the degree of coagulation, describe briefly the three most common methods?

- What is a roller pump and what are the main advantages/disadvantages?

- What is a centrifugal pump, and what are the main advantages/disadvantages?

- What is a “priming volume”?

- Would you use a roller pump or a centrifugal pump for a patient with a 24-hour need for assisted circulation?

- Why is it easy to calculate the flow with a roller pump, as opposed to a centrifugal pump?

- What is an oxygenator and what are the main parts?

- For a Heart-Lung Machine (HLM), is the venous reservoir open to air or a closed system, and why?

- What are the main risks with the HLM?

- If you should give cardiac support to a patient for some days, would you use Veno-arterial ECMO of Veno-venous ECMO

- How is it possible to make a medical suction based on over-pressure from the wall inlet?

- EXPERT: What would happen if the left ventricular vent tube were incorrectly mounted?

Chapter 12

- Describe briefly the main parts of a neuron, how can a neuron stimulate other neurons?

- Which cells in the brain contribute to the EEG-signals?

- What is the difference between temporal and spatial summation of neurons?

- Why is it important with myelin?

- What is the main problem with scalp electrodes?

- How can you make a tangential EEG measurement and how can you measure a radial EEG measurement

- What is the most important requirement for an EEG-amplifier?

- Why is the Nyquist theorem important for EEG-measurements?

- What is the Nernst equation actually describing?

- What does it mean that a cell is in a volume conductor and what is the problem with that?

- Which nerve fibers will give the largest signal?

- What is a reflex, and why is it important?

- How can you measure neural velocity?

- When measuring the neural velocity in the median nerve (in the hand), how would you set up the equipment and why is the signal at the elbow so different?

- What is an H-reflex, and how can we measure it?

- What is EMG and what is it used for?

- What is MEG, and how does it work?

- How can you measure a MEG-signal?

- Why is the fMRI preferred at the expense of MEG?

- What is ERG, how can it be measured and what can you get out of these measurements?