FYS 4250

Kapittel 13

- What is ultrasound?

- How can ultrasound be used for medical imaging?

- What is the relation between ultrasound frequency and attenuation, and what determines the frequency in an ultrasound system?

- What is a piezo crystal and how is it used in an ultrasound system?

- How can we form the ultrasound beam in order to focus at a certain distance?

- What is a phased array transducer and what is dynamic receive focusing?

- What is in order to maximize axial resolution?

- What is important in order to discriminate two objects perpendicular to the beam?

- What determines the axial, elevational and lateral resolution?

- What is the relation between ultrasound transducer frequency vs resolution when it comes to penetration depth and resolution?

- Describe briefly the most common ultrasound modi and give examples of the different applications for each of them

- What is time-gain compensation and why is it important?

- What are the main sources of error in a regular ultrasound system?

- What is Doppler ultrasound and what is it used for?

- What is the problem with Doppler ultrasound aliasing?

- What are the main advantages/disadvantages with ultrasound?

- What is focused ultrasound and what is it used for?

- What are the main advantages/disadvantages with focused ultrasound compared to traditional surgery?

- EXPERT: What frequency will compress and decompress the medium longest, 2 MHz or 5 MHz?

- What is spatial resolution and which factor will not influence on spatial resolution?

  - Frequency

  - Pulse interval

  - Time-gain compensation

  - Transducer thickness

Kapittel 14

- What is the frequency ranges for the electromagnetic radiation energy from an MRI-imaging system, compared to an X-ray system?

- What are the basic physic properties we take advantage of when making an MRI-image?

- What is the Larmour-frequency and why is it important in MRI-imaging?

- What is T1 relaxation and explain why you need several different delaytimes between the two 90 degree pulses in order to find T1?

- What is T2 relaxation and what is spin-echo contrast weighting?

- EXPERT: What is the difference between T2 and T2*, and why is T2 always longer than T2*

- How can we find the T2-spin echo decay?

- Why is it important with a gradient magnetic field, and how can we apply such a field?

- How is the stationary magnet designed in order to produce the gradient magnetic fields?

- What is a frequency encode gradient (FEG) and how can we use that in order to obtain spatial image information?

- What is the task of a shimming coil?

- What is functional MRI and how is it possible to use the MRI-scanner for the functional image?

- Describe a couple of MRI-artefacts and explain why they appear

- What kind of method is used in the MRI-scanner for picture reconstruction?

- What are the main advantages/disadvantages with MRI?

- What are the main concerns when it comes to safety issues related to the MRI?

- What is a slice select gradient (SSG) and how do we use it?

- EXPERT: If the total signal of pixel A and B with the same encoded frequency is written:      S0(t) = (A+B) sin wt, does it mean that phase-encoding gradient has been applied or not?

- EXPERT: What is better with a 3T MRI scanner compared to a 1T scanner, and what is better with the 1T scanner when it comes to image quality?

- EXPERT: What happens if two different gradients are applied simultaneously?